# Appendix 11 - Lead Diver Checklist

Below is a checklist designed to help OSU Lead Divers while in the field. It is not intended to be a comprehensive list of responsibilities and should be supplemented by specifics for project and location. OSU DSO should be notified of serious problems immediately and be debriefed upon cessation of diving.

## Pre-Dive

### General

- Environmental assessment: ‘GO or NO-GO’: (currents, tides, sea state, visibility, water temp, weather forecast)

- Gear checks (equipment function, adequate breathing gas, gas supply fully open, mission-specific equipment)

- Emergency Plan onsite with local emergency contact info and diver emergency contact info

- Location and type of available safety equipment

- Dive Briefing (turn pressure/ending pressure, dive time limits, depth limits. Review: hand signals, buddy and task assignments, diver separation procedures, verify diver fitness).

- Project Dive Log pre-dive areas completed

### Boat Dives

- Operator/Captain and Crew identified; roles/responsibilities discussed; topside dive tender recommended

- Location of safety equipment (First Aid, O2, fire extinguishers, floatation aids, dive flag deployed onsite)

- Radio operation verified; back-up radio and back-up GPS recommended w/spare batteries

- Adequate fuel

- Float plan on file and overdue check-in procedure confirmed

- Ensure proper dive gear storage, especially cylinders; consider vessel stability and capacity

- Review procedures for entering the water and re-boarding the boat (confirming boat in Neutral)

- Live-boat procedures reviewed; diver recall signals reviewed

- Mooring: double-check mooring line attachment to vessel before entering the water

- Anchoring: verify anchor is set properly underwater at start of dive if applicable

### Shore Dives

- Overdue check-in procedure confirmed

- Review underwater navigation for return to exit point

- Deployment of float/dive flag

## Post-Dive

- Diver roll call and wellness check

- Secure samples and/or data

- Complete Project Dive Log

- Debriefing (recap dive, review problems or issues, review ending pressures).

- Review surface interval activities (repetitive dive planning, swap/refill cylinders, manage body temp. and hydration, science tasks)

- Close float plan (if boat diving) or check-in with shore contact (if shore diving) at end of operations

- Debrief DSO upon return to OSU or as issues arise in the field